Score:_______  Section:_____Name:______________________________

# ECE 3055 Homework 2 – Due February 27

A direct mapping cache with WTWA (i.e. Write Through Write Allocate) is used to reduce the average memory access time on a computer.  This cache holds 256 entries, 16 bit data, and ties into a twenty-bit address bus with no byte or word select bits.  The block size is one word. Determine the contents of the cache and memory below after the memory requests shown are sent to the cache.  Keep track of hits and misses so you can compute the cache miss rate when you are done.  All values are in hex.

Cache before			Cache after

Block	Tag	Data		Block	Tag		Data

1	20	FFFF		1	______	____________

2	50	CABE		2	______	____________

3	500	BABA		3	______	____________

4	20	FACE		4	______	____________

5	5	0001		5	______	____________

Memory before		Memory after

Address	Data			Address		Data

20004	FACE			20004		_____________

50003	BABA			50003		_____________

40001	1234			40001		_____________

40004	CABE			40004		_____________

- Memory requests

Address		Type	Data		Hit(y/n)

20004		Read			____

40001		Write	0CAE

50003		Write	0000

50003		Read			____

20003		Read			____

40001		Read			____

40004		Read			____

- Cache hit rate (include reads only) = ______________%