Score:__________   Name:_________________________________________________

- ECE 3055 Test II

- (5%) A virtual memory system has 32-bit virtual addresses and a page size of 2048 bytes. If each entry in the page table is 4 bytes, what is the maximum size of the page table?

Page Table size  = ________________(in bytes)

- (5%) Why does a very small TLB achieve a very high hit rate?

- (5%) What is the average time to read or write a sector on a 10,000RPM disk drive with an average seek time of 10ms, a transfer rate of 10.24  MB/sec, and 1024-byte sectors. Assume the disk is idle and there is a 1ms controller overhead per sector.

Average R/W time per sector  =______________(in ms)

- (5%) Which type of bus (synchronous or asynchronous) would work best in a system where the bus length and loading could vary widely depending on the system options and configuration? (explain).

- (5%) The latest PCI bus can support 64-bit data and a 66Mhz clock. In burst mode, one clock is required per transfer cycle. What is the maximum possible bandwidth in Mbytes per second?

Bandwidth = _______________(in Mbytes/second)

- (5%) Where would you expect to find queues inside an Operating System?

- (5%) What are the tradeoffs in selecting the value of the time slice in a multiprogramming operating system?

- (5%) What is the difference between threads and processes?

- (5%) What can you conclude about deadlock in an unsafe state?

- (5%) Using the Text’s notation, add a semaphore called A to the processes below so that they will function correctly with the shared variable V. V is the only shared variable and all references to V are shown. Semaphore A does not need to be declared in each process.

P0				P1			P2

.				.			.

.				.			.

.				.			.

.				.			.

.				.			.

IF V=W Then V=X;		If V=Z then V=Y;	.

.				.			.

.				.			.

.				.			.

.				.			.

Set Associative Cache Memory

11. (10% )A two-way set associative cache is given the memory operations shown below.  It contains eight lines and has one word per block.  A LRU (Least Recently Used) replacement policy is used on this cache.  The cache is initially empty.  The first column of  the cache gets the first entry after the valid bits have been reset.  In the blanks provided in the Cache below, list the original address value only (i.e. 12 in blank, means address and data for memory location 12 is in cache).

Memory Read Operations

Address  Hit (Y/N)?

2	___						Cache Contents

4	___					Block		Element 1	Element 2

8	___					0		_______		________

5	___					1		_______		________

20	___					2		_______		________

17	___					3		_______		________

19	___					4		_______		________

64	___					5		_______		________

8	___					6		_______		________

11	___					7		_______		________

4	___

43	___

2	___					Hit rate for all memory operations is __________(%)

6	___

9	___

917	___

Virtual Memory

12. (15%) Assume a 16-entry direct-mapped TLB is used in a system with 20-bit virtual byte addresses, 16-bit physical byte addresses, and 256 byte pages.  For the sequence of virtual addresses shown below, state whether each address causes a TLB hit or miss and show the physical address generated. Portions of the initial contents of the TLB and full page table are given.  Assume all listed page table entries are valid.  Show the final contents of the TLB after this sequence of addresses is accessed.

- Virtual address sequence

Address		Hit/Miss		Physical Address

00013			_______		______________

022CF			_______		______________

023A1			_______		______________

00101			_______		______________

000FF			_______		______________

02015			_______		______________

02266			_______		______________

TLB Initial Contents					TLB Final Contents

Block		Tag		Data			Block		Tag		Data

0		   02		  71			    0		____		____

1		   02		  C0			    1		____		____

2		   00		  55			    2		____		____

3		   20		  E2			    3		____		____

- Page Table Initial Contents

Virtual Page #				Data

000				  20

001				  F0

002				  55

003				  30

…

020				  71

021				  C0

022				  A0

023				  E2

#### CPU Scheduling

13. (15%) The following processes arrive for execution at the times indicated. Use preemptive scheduling and base all decisions on the information you have at the time the decision is made. At arrival time, the execution time is known.

## Process		Arrival Time		CPU Execution Time

##### P1			0.0ms			8ms

P2			0.4ms			4ms

P3			1.0ms			1ms

Draw a Gantt chart using FCFS:

_______________________________________________________________________________

|                                                                                                                                                              |

|_______________________________________________________________________________|

What is the average turnaround time for these processes with FCFS? _______________

Draw a Gantt chart using SJF:

_______________________________________________________________________________

|                                                                                                                                                              |

|_______________________________________________________________________________|

##### What is the average turnaround time for these processes with SJF?   _______________

Deadlock Avoidance

14. (10%) The following states are in effect:

### Process	Allocation	Max		Need		Available

### ABCD		ABCD		ABCD		ABCD

P0		0 0 1 2		0 0 1 2		0 0 0 0 	1 5 2 0

P1		1 0 0 0		1 7 5 0		0 7 5 0

P2		1 3 5 4		2 3 5 6		1 0 0 2

P3		0 6 3 2		0 6 5 2		0 0 2 0

P4		0 0 1 4		0 6 5 6		0 6 4 2

Process P1 issues a request for (0,4,2,0) can the request be granted?

(justify your answer by showing your work below and show a safe execution sequence, if one exists)

Process	Allocation	Max		Need		Available

ABCD		ABCD		ABCD		ABCD

P0		0 0 1 2		0 0 1 2

P1				1 7 5 0

P2		1 3 5 4		2 3 5 6

P3		0 6 3 2		0 6 5 2

P4		0 0 1 4		0 6 5 6

Grant Request     Y______        or  N_________

Safe execution sequence = {_____________________________}