### Score:_______   Date:_____   Name:__________________________ Section: ________

### TA Signoff:_______________________________________

- ECE 3055 Laboratory Assignment 1

- Due Date: Tuesday, September 8

- Write a MIPS assembly language program that computes the parity of a number you type in.  Your program should prompt the user to enter an integer n, and then calculate and print the result stating the “The parity value of n is even” or “The parity is value of n is odd”.  You can assume that n is a positive integer such that it is small enough to be represented by a 32-bit 2’s complement number. The program code that calculates the parity must be in a subroutine (i.e JAL to Call from main program, JR $31 to Return). Pass and return values to the subroutine in registers. $a0 is the number to compute the parity of, and $a1 returns a flag to the main program. The flag value is 1 for even parity and 0 for odd parity. Keep all I/O operations in the main program and you must save the value of n in data memory prior to calling the subroutine and when returning from the subroutine you must read the value of n back from data memory. (Note: This will force your short program to have a LW and SW example even though there are enough registers to avoid it.)

- To compute the parity (number of 1’s in the 32-bit binary number) of the number in $a0 use a loop (32 times for 32-bits) in the parity subroutine that checks one bit at a time using single right bit shifts, and a mask operation with an logical AND to throw away all of the other bits, and then a test for 1 or 0 in the bit. If it’s 1, increment a register that counts number of 1’s encountered. After the loop exits, if the low bit of this count register is 0 (i.e. a mod 2 operation), you have an even number of 1’s or even parity and can set $a1 to 1. If you need any extra registers other than $a0 or $a1 in the parity subroutine, you must save and restore them using stack operations in the subroutine (Note: this is a somewhat artificial requirement, but it forces you to include and understand some basic MIPS stack operations even in a short program that would not run out of registers).

- Use the SPIM simulator running on the lab PCs or your home PC to implement the program.  The program must be demonstrated in the Klaus PC labs to one of the 3055 TAs for credit. See the Lab hour web page for TA lab times. For students with home PCs, SPIM and the optional Crimson editor executable can be downloaded from the lab’s Web page (http://www.ece.gatech.edu/~hamblen/3055 ).

- Recall that Appendix B of the Patterson and Hennessy text contains information on SPIM and SPIM’s I/O function calls. Chapter 2 contains MIPS assembly language program examples. See the example program from class for some I/O and loop examples on the web at http://users.ece.gatech.edu/~hamblen/3055/course/hwk10.s - you may find it useful to use it as an initial template or starting point. It has the additional assembler directives that are needed for a complete SPIM program.