Score:_______  		Name:______________________________

# ECE 3055 Quiz - April 13, 2011

- A ____context______________  ____switch___________ is the name for the task in an Operating System that saves the process state to stop running a process, and restores it to run a different process on the CPU. It does not include the code with the scheduling algorithm that decides which process to run next.

- How would code in an application typically (i.e., not an error) do something that causes the mode bit to change?

OS API call that jumps to kernel code

- List the three most common types of events (i.e., not errors and likely to occur in any application) that will move a process out of the CPU run state.

- Time slice interrupt

- API call for I/O

- Process Terminates

- Which OS queue would a long I/O  bound process on a system that is not running other CPU intensive processes be likely to spend the majority of its time in the system  (explain)?

I/O wait queue – it would spend the majority of the time waiting for slow I/O devices and not the CPU.

- In a modern OS, what hardware feature would stop a program running with an infinite loop error so that other process can also run? (Explain including a short functional description of how it would be physically implemented using common digital hardware components)

A time slice interrupt would occur and stop the process. In hardware, a programmable count down timer is used that generates the interrupt signals when the count reaches zero. The counter is reset each time a new process starts running.