Score:____________Name:_________________________________________________

ECE 3055 A Quiz 1 – Fall 2008

The following RISC assembly language program is executed on a MIPS processor. Fill in the register values that will be present, after execution of this program. A summary of MIPS instructions is included at the bottom of the page – for anyone unfamiliar with the MIPS instruction set. Prior to execution of the program, memory location 0x01000 contains 0x20313055. Note: 0x indicates hexadecimal and all answers must be in hexadecimal, default is decimal in the MIPS assembly language source file. A MIPS memory word or register contains 32-bits. Use XXXXXXXX for an undefined value.

# LW	  	$3, 0x01000

SRL		$4, $3, 8

ADD		$3, $4, $3

XOR		$2, $3, $4

LUI		$5, 0x3035

ORI		$5, $5, 63

# SUB		$6, $3, $4

BGT		$6, $2, LABEL1

# ADDI		$6, $0, -4

### LABEL1:	SW		$6, 0x01000

After execution of the MIPS code sequence above,

R2 = 0x207150b5______________( in hexadecimal)

R3 = 0x20516185______________( in hexadecimal)

R4 = 0x00203130______________( in hexadecimal)

R5 = 0x3035003F______________( in hexadecimal)

Memory Location 0x01000 contains:  0xFFFFFFFC______________( in hexadecimal)

The MIPS processor contains thirty-two 32-bit registers, $0 through $31. $0 always contains a zero. By default, all arithmetic operations use two’s complement arithmetic. Assume no branch delay slot is present.

## MIPS Instruction				Meaning

ADD   	Rd, Rs, Rt		-	Rd = Rs + Rt  (R – register ($) )

AND   	Rd, Rs, Rt		-	Rd = Rs bitwise logical AND Rt  (R – register ($) )

ORI	Rd, Rs, Immed		-	Rd = Rs bitwise logical OR Immediate value

LUI	Rd, Immed		-	Rd = 16-bit Immediate value high 16-bits, 0’s low 16-bits

BNE	Rs, Rt, address		-	Branch to address, only if Rs not equal to Rt

LW	Rd, address		-	LOAD - Rd gets contents of memory at address

SRL	Rd, Rs, count		-	Shift right logical (use 0 fill) by count bits

SUB	Rd, Rs, Rt		-	Rd = Rs - Rt

SW	Rd, address		-	STORE - memory at address gets contents of Rd

XOR	Rd, Rs, Rt		-	Rd = Rs bitwise logical XOR Rt