The following RISC assembly language program is executed on a 32-bit MIPS processor. Fill in the register values that will be present, after execution of this program. A summary of MIPS instructions is included at the bottom of the page – for anyone unfamiliar with the MIPS instruction set. Prior to execution of the program, memory location 0x02000 contains 0x20313055. Note: 0x indicates hexadecimal and all answers must be in hexadecimal, default is decimal in the MIPS assembly language source file. A MIPS memory word or register contains 32-bits. Use XXXXXXXX for an undefined value.

# lw	  	$3, 0x02000

sll		$4, $3, 10

sub		$2, $4, $3

xor		$3, $4, $2

lui		$5, 0

ori		$5, $5, 12373

# add		$6, $4, $3

bne		$3, $6, LABEL1

# addi		$6, $0, -20

### LABEL1:	sw		$6, 0x02000

After execution of the MIPS code sequence above,

R2 = 0x__A49023AB______________( in hexadecimal)

R3 = 0x__605177AB_______________( in hexadecimal)

R4 = 0x__C4C15400_______________( in hexadecimal)

R5 = 0x__00003055________________( in hexadecimal)

Memory Location 0x02000 contains:  0x____2512CBAB____________( in hexadecimal)

The MIPS processor contains thirty-two 32-bit registers, $0 through $31. $0 always contains a zero. By default, all arithmetic operations use two’s complement arithmetic. Assume no branch delay slot is present.

## MIPS Instruction				Meaning

ADDI	Rd, Rs, Immed		-	Rd = Rs + Immediate value

ADD   	Rd, Rs, Rt		-	Rd = Rs + Rt  (R – register ($) )

ORI	Rd, Rs, Immed		-	Rd = Rs low 16-bits  bitwise logical OR Immediate value

LUI	Rd, Immed		-	Rd = 16-bit Immediate value high 16-bits, 0’s low 16-bits

BNE	Rs, Rt, address		-	Branch to address, only if Rs not equal to Rt

LW	Rd, address		-	LOAD - Rd gets contents of memory at address

SLL	Rd, Rs, count		-	Shift left logical (use 0 fill) by count bits

SUB	Rd, Rs, Rt		-	Rd = Rs - Rt

SW	Rd, address		-	STORE - memory at address gets contents of Rd

XOR	Rd, Rs, Rt		-	Rd = Rs bitwise logical XOR Rt