SCORE:________		Name:__________________________________________

ECE 4100 Advanced Computer Architecture

#### Final Exam – Summer 2003

- (10 points) What is the significance of a “snooping protocol” and what is it used for in modern computer systems?

- (10 points) What limits the size and complexity of the L1 cache and why is an L2 common today?

3. (5 points) What are the advantages and disadvantages of FLASH memory over Disk?

4. (10 points) Adding 3 additional processors to a parallel computer system runs an application 3.5 times faster than on one processor (ignore additional communication overheard between processors and any possibility of superlinear speedup effects).  What percentage of the original program code would have to be able to run in parallel on four processors for this to be possible? (assume it only runs parallel on four or sequential on one)

Percentage of code that can run in parallel _____%

5. (10 points) Assume a network with a 100M bit/sec bandwidth has a sending overhead of 50 usec. and a receiving overhead of 75 usec. How long would it take to send a 1Mbyte message. Assume the machines are 5000 km apart and use the book’s speed of light in a conductor estimate (2/3 of the speed in a vacuum). Compute the total latency for the message (to four decimal places).

Total latency for message  _____________________

6. (10 points) Compute (to four decimal places) the average time to read or write a 4096-byte sector on a disk with these features:

Average seek time is 8ms (use the book’s suggested 1/3 correction factor for a more realistic seek time) a transfer rate of 20 Mbytes/sec, the disk rotates at 15,000 RPM, and the controller overhead is .1ms

Average time to read or write a sector is ___________ms

7. (10 points) A processor sends 50 disk I/Os per second, the I/O requests are exponentially distributed, and the disk drive has an average service time of 18ms. Use queuing theory to compute the following:

Disk utilization _________%

Average Queue Time = _____________ms

Average Queue Length = _______________

8. (10 points) A computer has two levels of cache. The L1 cache has a 5% miss rate and the L2 cache has a local miss rate of 33%. Main memory takes 50 clock cycles at 2Ghz, an L2 hit is 8 clock cycles, and an L1 hit is 1 clock cycle. Compute the average memory access time.

Average memory access time = _____________________________ ns.

9. (15 points) Part I (8 of 15 points): Unroll the loop shown below three times to reduce the number of stalls and control overhead. You can assume that the loop executes a multiple of three times. Use registers F10..F30, if needed. Indicate any stalls in your answer.

| Instruction producing result | Instruction using result | Latency in clock cycles |
| --- | --- | --- |
| FP ALU Op | FP ALU Op | 3 |
| FP ALU Op | Store Double | 2 |
| Load Double | FP ALU Op; | 1 |
| Load Double | Store Double | 0 |

LOOP: 		L.D 	F8, 0(R1)

L.D 	F4, 0(R2)

ADD.D	F6, F4, F8

SUB.D 	F4, F6, F10

S.D	F4, 0(R1)

DADDIU R1, R1, #8

BNE  	R1, R3 LOOP

Part II (7of 15 points): Using the code example above (with the same number of loop un-rollings as a basis), use software pipelining to minimize stalls. Startup and cleanup code is not required. Indicate any stalls in your answer. Note: Do not unroll more than 3 times for the pipeline code.

10. (10 points) Consider the program segment below running on a single-issue machine using Tomasulo’s Algorithm. Fill in the clock cycle number in the table below assuming the latencies shown in the table below the program.

L.D 		F4, 0(R1)

L.D 		F6, 0(R2)

MUL.D 	F2, F6, F4

SUB.D 	F4, F4, F10

### DIV.D  	F4, F6, F2

ADD.D 	F8, F8, F4

DADDIU 	R1, R1, # -8

BNE  		R1, R3 LOOP

Details of Functional Units:

| Unit | Latency (in Execute) | Reservation Stations |
| --- | --- | --- |
| FP Add/Sub | 5 | 2 |
| FP Mult | 8 | 2 |
| FP Div | 12 | 2 |
| FP Load/Store | 2 | 2 each Load/Store buffers |
| Integer Unit | 1 | None |

Note: The FP arithmetic units are NOT pipelined – i.e. you must wait for the current operation to finish execution before using the unit again. You can do a new FP load/store every clock cycle. The WB stage takes only 1 clock cycle and during that cycle the new data appears on the CDB and at all reservation stations.

| Instruction | Issue | Execute | WB |
| --- | --- | --- | --- |
| L.D F4, 0(R1) | 0 |  |  |
| L.D F6, 0(R2) |  |  |  |
| MULT F2, F6, F4 |  |  |  |
| SUB.D F4, F4, F10 |  |  |  |
| DIV.D F4, F6, F2 |  |  |  |
| ADD.D F8, F8, F4 |  |  |  |
| DADDIU R1, R1, #-8 |  |  |  |
| BNE R1, R3 LOOP |  |  |  |

Optional Worksheet for last problem, use it only if you find it helpful to keep track of everything (this sheet will not be graded and does not need to be turned in)

##### Reservation Stations

| Name | Busy | Op | Vj | Vk | Qj | Qk | ###### A |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Load1 |  |  |  |  |  |  |  |
| Load2 |  |  |  |  |  |  |  |
| Add1 |  |  |  |  |  |  |  |
| Add2 |  |  |  |  |  |  |  |
| Mult1 |  |  |  |  |  |  |  |
| Mult2 |  |  |  |  |  |  |  |
| Div1 |  |  |  |  |  |  |  |
| Div2 |  |  |  |  |  |  |  |

##### Register Status

| Field | F2 | F4 | F6 | F8 | F10 |
| --- | --- | --- | --- | --- | --- |