Score:____________   Name: _________________________________

CmpE 2510 Computer Architecture

Test II     March 1, 1996

Place all  answers in the blanks provided.

1. (5 points) Convert - 151.375, a decimal number, to IEEE 32-bit floating point format.

IEEE Hex Number = ______________________

2.   (5 points) Which MIPS computer has a more complex control unit, the single cycle implementation or the multi-cycle implementation?

3.  (5 points )  In VHDL, which statement other than IF, Select, or Case could be used to model a mux.

4.  (5 points ) In the MIPS  single clock cycle  model there is only one place other than memory or the register file that contains D flip-flops, where is it?

5. (10 points) Perform the unsigned integer multiplication shown below using Booth's Algorithm.  Show your work. Include all partial product terms.  Place the partial product terms  and the final product in the blank lines provided. No credit for answer only!

1 1 0 0 1

x  0 1 1 1 0  (number that is to be recoded)

___________

product

1   1   0   0   1

x

___________________________(in Booth’s recoded form)

_____________________________________

_____________________________________

_____________________________________

_____________________________________

_____________________________________

_____________________________________

_____________________________________

6.  (10 points) Compute the square root of 99 using the algorithm from lab 3 which is accurate to 24-bits of fraction.    After computing the fraction, Put the decimal value of intermediate variables from the algorithm in the spaces provided - No credit for answer only!  Intermediate calculations can be done in decimal using a calculator.  The final answer should be correct to at least three decimal places.

f = ______________________

N = ______________________

y0 = _____________________

y1 = _____________________

y2 = _____________________

M = ______________________

(99)1/2 = __________________  (binary floating point value-show fraction and exponent)

(99)1/2 = __________________  (decimal value)

7.  (15 points) Use the text's algorithm and multiply the two IEEE 32-bit numbers shown below. Show your work for the exponent and fraction operations required - No credit for answer only!

C0D0000016 divided by BFD0000016 = _______________________16

Exponent Calculations Below		Fraction Calculations Below

8.  (30 points ) The instruction, sw $3,0x0001, is executed on the MIPS single clock cycle model.  The registers are loaded with the register number prior to execution and memory location 1 (i.e. actual address is 4 after two-bit shift) contains 0x000000A5.  Fill in the values of the following control signals and buses.

MIPS BUS values in hexadecimal

Instruction Bus = _________________________

Register file Read Data Bus 1 = ______________________

Register file Read Data Bus 2 = ______________________

ALU result Bus = _____________________________

Write Back Data Bus = _________________________

MIPS Control Signals in Binary

Mem Write = _________

Reg Write = _________

Reg Dst = ________

AluSrc = _________

ALU Op (two-bits) = ______________

ALU control (three bits) = _____________

9.  (15 points ) Add the instruction, JR, to the original MIPS VHDL synthesis model.  First, describe any hardware changes needed, then show the changes required to the various VHDL modules.  You can assume the Jump instruction hardware on page 307 has already been added.