Score:________  Name:_____________________________________________

CmpE 2510 Final Exam

Test is open book and notes.  Place all answers in the spaces provided.

No credit for answers not in blanks.

Short Answer questions (20 points -2 points per problem number)

1.   A ideal pipeline has 5 stages, for n=3 it would be __________ times faster than without a pipeline.

2.  Show how you could shift the 8-bit signal, Aluout, two bits left without using the SHIFTx function in VHDL.

3,4.  Convert -1510.1, a decimal number, to IEEE 32-bit floating point format.

IEEE Hex Number = ______________________

5.  The pseudo op, ____________ , is used to reserve a 32-bit location in memory for a single precision floating point program variable.

6-10.  In the space provided below, perform the operation 5 - 6 in binary, using two's complement arithmetic with four bits only.

(5)

(+(-6) in two's complement)

+__________________

=			(answer)

For this operation carry = _________ and overflow = ______________

11. (10 points) The following sequence of MIPS instructions is executed.  Indicate the resulting register values in the spaces provided below.  Prior to execution of this sequence $0=0, $2=0x73334444, $6=0x7555AAAA and memory contents of address, label1=0x87654321.  All numbers preceded by 0x are in hex with leading zeros omitted.  Numbers without 0x are in decimal.  All answers must be in hex.

slt	$3,$2,$6

bne	$3,$0,snafoo

xori	$2,$0,123

snafoo:	addi 	$4,$2,-4

ror	$6,$6,4

lui	$4,0x4321

ori	$4,$4,0x5678

addi	$5,$4,0xCABE

lw	$7,label1($0)

$0 = _____________________

$2 = _____________________

$3 = _____________________

$4 = _____________________

$5 = _____________________

$6 = _____________________

$7 = _____________________

memory contents of address, label1 = ________________________

12.  (10 points) Write a complete SPIM program that implements the following C code.  The program must include all required data declarations and assembler directives.  Labels must be setup and used for a, b, and c.  Registers $16,$17,$18 should be used for the values of b, c, and i.  The program must add the values of a and b obtained from memory.  The new value of c must be written back to memory.  For full credit, use the minimum number of instructions required.  Subroutine, zappo, has no arguments and is defined elsewhere, do not include code for the subroutine definition or to save $31 on the stack, in your answer, only the call is needed.

int x=7, y,i;

float a[7];     /* also assigns initial value 7, to  b, a is an flt. point array*/

{

zappo();                    /* subroutine call to zappo */

/* assume zappo loads a array and i */

y=(x+1)/i<<2;

printf("%f",a[y+1]);      /* prints c in floating format on console */

}

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

13.  (5 points) Implement the following C control structure statement using MIPS instructions.  Use these register assignments, x times 4,y, and z, are already loaded in $16,$17, and $18.  Use only $8 for temporary data storage, if needed.  The new value of y must be placed back into memory by your program.

For full credit, use  the minimum number of instructions required.  Assume the labels for w, x, y, and z and subroutine foobar are already declared, do not include them in the answer below.

if ((w[x]-1)<=0) foobar; else y=z[x];

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

_________________________________________________________________________________

14.  (5 points)  A VLSI technology has up to five inputs per gate.  How large can the lookahead carry groups be made?  How would you build a 32-bit adder? Justify your answer.

15..  (5 points) Compute the square root of 2510 using the algorithm from lab 3 which is accurate to 24-bits of fraction.    Put the decimal value of intermediate variables from the algorithm in the spaces provided - No credit for answer only!  Intermediate calculations can be done in decimal using a calculator.  The final answer should be correct to at least three decimal places.

f = ______________________

N = ______________________

y0 = _____________________

y1 = _____________________

y2 = _____________________

M = ______________________

(2510)1/2 = __________________  (binary floating point value-show fraction and exponent)

(2510)1/2 = __________________  (decimal value)

16. (15 points) Use the text's algorithm and add the two IEEE 32-bit numbers shown below. Show your work for the exponent and fraction operations required - No credit for answer only!

42C8000016 plus C120000016 = _______________________16

Exponent Calculations Below		Fraction Calculations Below

17. (15 points ) The instruction, sub $1,$4,$3, is executed on the MIPS single clock cycle model.  The registers are loaded with the register number prior to execution.  Fill in the values of the following control signals and buses.

MIPS BUS values in hexadecimal

Instruction Bus = _________________________

Register file Read Data Bus 1 = ______________________

Register file Read Data Bus 2 = ______________________

ALU result Bus = _____________________________

Write Back Data Bus = _________________________

MIPS Control Signals in Binary

Mem Read = _________

Reg Write = _________

Reg Dst = ________

AluSrc = _________

ALU Op (two-bits) = ______________

ALU control (three bits) = _____________

18.  (15 points ) Add the instruction, SLTU, to the original MIPS VHDL synthesis model.  First, describe any hardware changes needed, then show the changes required to the various VHDL modules.