Score:___________      Name:______________________________________________________

Cmpe 2510 Homework 1

Due Date:Wednesday, January 29

The following sequence of MIPS instructions is executed.  Indicate the resulting register values in the spaces provided below.  Prior to execution of this sequence $0=0, $6=A3334444, $2=5AAA5555 and mem(100)=12345678.  All numbers are in hex with leading zeros omitted.

slt	$3,$2,$6

beq	$3,$0,foobar

ori	$2,$2,0x77

foobar:	addi 	$4,$0,-4

sll	$6,$6,0x8

lui	$4,30

addi	$5,$4,0x19

sw	$5,0x100($0)

$0 = _____________________

$2 = _____________________

$3 = _____________________

$4 = _____________________

$5 = _____________________

$6 = _____________________

MEM(100) = ________________________