Score:________    Name:__________________________________

CmpE 2510 Homework 2

Due Date: Friday, May 13

1.  Add 1.00 to 1.500 using IEEE 32-bit floating point using the algorithm given in the text.  Show all work in binary for the fraction and decimal for the exponent.  In the binary fraction calculations it is not necessary to show all the trailing zeros.

1.000 = _______________ IEEE 32-bit hex format (Do not show work)

1.500 = _______________ IEEE 32-bit hex format (Do not show work)

2.500 = _______________ IEEE 32-bit hex format (Result from work below)

Exponent Calculations in Decimal	Fraction Addition in Binary

Note: Include IEEE Bias			Note: Include implicit "1" bit

(exp 1.0)						     (frac 1.0)

(exp 1.5)

(frac 1.5)

-

+

Shifts Required to

align fractions   = ____

(prior to addition)

Shifts Required for Normalization Step =

IEEE Fraction of Final Result = _________________________

(without implicit "1" after normalization in hex)

IEEE Exponent of Final Result = ___________

(after normalization and includes IEEE Bias in decimal)