Score:__________   Name:_________________________________________________

CmpE 3510 Final Exam

- (10 points) Fill in the numbers requested below for a parallel computer with “n” processors containing the indicated interconnection network built using current technology:

Network		Busses/Processor	   Max nodes/transfer                  Approx. range of n which makes sense

Single Bus

Crossbar

Hypercube

2D Mesh

- 2.(10 points). Set Associative Cache Memory A four-way set associative cache is given the memory operations shown below.  It contains eight lines and has one word per block.  A LRU (Least Recently Used) replacement policy is used on this cache.  The cache is initially empty.  Starting from the left,  the first available column of  the cache gets the new entry after the valid bits have been reset.  In the blanks provided in the Cache below, list the original address value only (i.e. 12 in blank, means address and data for memory location 12 is in cache).

Memory Read Operations

Address  Hit (Y/N)?

0	___						Cache Contents

4	___		Addr	Set 1		Set 2		Set 3		Set 4

8	___		0	_______		________	________	________

1	___		1	_______		________	________	________

5	___		2	_______		________	________	________

17	___		3	_______		________	________	________

16	___		4	_______		________	________	________

9	___		5	_______		________	________	________

13	___		6	_______		________	________	________

11	___		7	_______		________	________	________

24	___

43	___

4	___					Hit rate for all memory operations is __________

1	___

9	___

53,115	___

3. (15 points) The following sequence of MIPS instructions is executed.  Indicate the resulting register values in the spaces provided below.  Prior to execution of this sequence $6=33334444, $2=78885555 and mem(300)=12345678.  All numbers are in hex.

slt	$3,$2,$6

bne	$3,$0,foobar

ori	$2,$2,0x77

foobar:	addi 	$4,$0,-4

srl	$5,$6,4

lui	$4,0x21

addiu	$6,$4,0xACE

sw	$4,0x300($0)

$0 = _____________________

$2 = _____________________

$3 = _____________________

$4 = _____________________

$5 = _____________________

$6 = _____________________

MEM(0x300) = ________________________

4. (15 points) Assume the  MIPS fetch and data memory were each two stages on the pipeline rather than one.  This version of the MIPS would have 7 stages.  On the real MIPS this is closer to the way it works since these two stages with memory have long delay times.  The first stage has a pipeline register for decoded address select lines (contains gates that generate select lines for FF muxes) and the second has a pipeline register for the data out of the memory(contains muxes on FF output bits).  Discuss the hardware changes that would be required to the existing MIPS model, including the final version of control unit.  VHDL code is not required,  just outline the major changes required.  Comment on any changes to branch and data hazards  that would result from this change.

5. (20 points) The program below is executed on the 5 stage pipelined MIPS described in chapter 6.  Answer the following questions about this program.

loop:	lw	$7,100($0)

sub	$7,$7,$1

sw	$6,100($7)

or	$6,$7,$2

add	$5,$6,$7

beq 	$0,$5,loop

\

Part I (7 points)  If the control unit does not have any hazard detection, forwarding or branch flushing, rewrite the code sequence by adding the minimum number of NOP instructions to eliminate all potential data and branch hazards.  Assume other non NOP instructions follow the branch in the original code sequence above.

Part II (7 points)If the control unit has hazard detection with automatic stalls and automatic branch flushing without any forwarding unit,  determine the number of clock cycles required to complete the first loop execution (i.e. start at loop, branch back to loop, and start the lw instruction on the next clock) of the original code sequence.  Include the time required to fill and flush the pipeline.

If there were no hazards or branch flushing the program would require    ___________ clock cycles for execution.

But the program stalls or flushes the pipeline ____________ clock cycles so a total of __________ clock cycles is required for execution.

Part III (6 points)  If the control unit is improved by adding the forwarding unit as outlined in the text,  determine the number of clock cycles required to complete the first loop execution of the original code sequence.  Include the time required to fill and flush the pipeline.  Use the load improved forwarding unit.

If there were no hazards or branch flushing the program would require    ___________ clock cycles for execution.

But the program stalls or flushes the pipeline ____________ clock cycles so a total of __________ clock cycles is required for execution.

6.(10 points) Plot the speedup vs. number of processors for an application which has 50% parallel code and 50% sequential code.  Assume the parallel code can be divided evenly between any number of processors and assume the inter processor communication time is zero.  Include points in the plot for 1,2,3,4, and 8 processors.  Draw another curve with a dashed line that assumes each processor spends an additional 20% of its time transferring data to/from other processors (i.e. 20% overhead to exchange data between processors).  If any number of processors were used what is the ultimate limit to the speedup (with and without communication time?

Max Speedup = __________

Max Speedup with 20% communication overhead = ____________________

S	|
p	|	
e	|	
e	|
d	|
u	|
p	|
 	|
 	|
 	|
 	|
 	|
 	|
 	|_____________________________________________________________

1  	2  	3        	  4		         	8

number of processors

.

VHDL Synthesis

7.(20 points) Write a complete viewlogic VHDL synthesis module for the cache hardware shown in the block diagram on page 509 of the text.  Include a synthesis command file and a simulation command file to test your module.