CmpE 3510 Computer Architecture II

Programming Assignment 3

LAB 3  Due Friday, May 9

(90 points maximum)

LAB 1 plus forwarding control.  Test your VHDL model by  running a simulation of the example program shown in figures 6.45-47. Note: The first hazard unit is not needed so start with your lab1 VHDL files and it will be easier.

Use the following test program

sub	$2,$1,$3

and	$4,$2,$5

or	$3,$2,$6

add	$5,$4,$2

slt	$1,$6,$7

Optional for LAB 3

(110 points maximum)

Add the Lw/Sw forwarding suggested in the text on page 423.