CmpE 3510 Computer Architecture II

Programming Assignment 4

counts as two programs

due Friday May 31

Option 1

(80 points maximum)

LAB 3 plus branch flushing as shown in figures 6.52 and 6.54 of the text.

Use the following test program

beq $0,$0,label1

add $1,$1,$2

add $2,$3,$0

label1: beq $1,$2,label1

add $1,$1,$2

add $2,$3,$0

nop

label2: beq $2,$1,label2

Option 2

(90 points maximum)

In addition to Option 1, add the overflow exception hardware suggested at the end of chapter 6 in figure 6.56.  Add an overflow circuit which produces the exception with a test program containing an add that overflows.  Show the PC and the trap address on your simulation file.  For test purposes make the exception address 6 instead of 40000040.

Option 3

(110 points maximum)

In addition to Option 2, redo option 1 assuming branch instructions have 1 delay slot (i.e. one instruction after the branch is executed even when the branch is taken).  Rewrite the VHDL and test program for option 1 assuming 1 delay slot.  You can move instructions around and add nops if needed.